INTERNET OF THINGS PROJECT

IOT PROJECT ON MINI WEATHER STATION

YEAR: 2020-21

DONE BY: R. DHARSHINII

REFERRED FROM:

IOT PROJECT ON MINI WEATHER STATION

WHAT IS A WEATHER STATION?

A weather station is a device that collects data related to the weather & environment using different sensors. There are two types of weather stations, one which is having own sensors and the second type of weather station is where we pull data from the weather station servers. In this tutorial, we will go for the first one, i.e. we will design our own weather station.

Weather station sensors may include a thermometer to take temperature readings, a barometer to measure the atmospheric pressure, Hygrometer to measure humidity, rain sensor to measure rainfall, an anemometer to measure wind speed, and more. Weather stations are also called weather centers, personal weather stations, professional weather stations, home weather stations, weather forecaster, and forecasters.

REQUIREMENTS:

- NODEMCU ESP8266

- BMP180 BAROMETRIC PRESSURE SENSOR

- DHT11 TEMPERATURE SENSOR

- FC-37 RAIN SENSOR MODULE

- RESISTOR (4.7 K)

- JUMPER WIRES

- BREADBOARD

CIRCUIT DIAGRAM:

PROGRAM CODE:

INDEX.H CODE

const char MAIN_page[] PROGMEM = R"=====(

<!DOCTYPE html>

<html>

<head>

<title>www.how2electronics.com</title>

</head>

<style>

@import url(https://fonts.googleapis.com/css?family=Montserrat);

@import url(https://fonts.googleapis.com/css?family=Advent+Pro:400,200);

*{margin: 0;padding: 0;}

body{

background:#544947;

font-family:Montserrat,Arial,sans-serif;

}

h2{

font-size:14px;

}

.widget{

box-shadow:0 40px 10px 5px rgba(0,0,0,0.4);

margin:100px auto;

height: 330px;

position: relative;

width: 500px;

}

.upper{

border-radius:5px 5px 0 0;

background:#f5f5f5;

height:200px;

padding:20px;

}

.date{

font-size:40px;

}

.year{

font-size:30px;

color:#c1c1c1;

}

.place{

color:#222;

font-size:40px;

}

.lower{

background:#00A8A9;

border-radius:0 0 5px 5px;

font-family:'Advent Pro';

font-weight:200;

height:130px;

width:100%;

}

.clock{

background:#00A8A9;

border-radius:100%;

box-shadow:0 0 0 15px #f5f5f5,0 10px 10px 5px rgba(0,0,0,0.3);

height:150px;

position:absolute;

right:25px;

top:-35px;

width:150px;

}

.hour{

background:#f5f5f5;

height:50px;

left:50%;

position: absolute;

top:25px;

width:4px;

}

.min{

background:#f5f5f5;

height:65px;

left:50%;

position: absolute;

top:10px;

transform:rotate(100deg);

width:4px;

}

.min,.hour{

border-radius:5px;

transform-origin:bottom center;

transition:all .5s linear;

}

.infos{

list-style:none;

}

.info{

color:#fff;

float:left;

height:100%;

padding-top:10px;

text-align:center;

width:25%;

}

.info span{

display: inline-block;

font-size:40px;

margin-top:20px;

}

.weather p {

font-size:20px;padding:10px 0;

}

.anim{animation:fade .8s linear;}

@keyframes fade{

0%{opacity:0;}

100%{opacity:1;}

}

a{

text-align: center;

text-decoration: none;

color: white;

font-size: 15px;

font-weight: 500;

}

</style>

<body>

<div class="widget">

<div class="clock">

<div class="min" id="min"></div>

<div class="hour" id="hour"></div>

</div>

<div class="upper">

<div class="date" id="date">21 March</div>

<div class="year">Temperature</div>

<div class="place update" id="temperature">23 &deg;C</div>

</div>

<div style="text-align: center;"><a href="https://www.how2electronics.com" style="align:center">www.how2electronics.com</a></div>

<div class="lower">

<ul class="infos">

<li class="info temp">

<h2 class="title">TEMPERATURE</h2>

<span class='update' id="temp">21 &deg;C</span>

</li>

<li class="info weather">

<h2 class="title">PRESSURE</h2>

<span class="update" id="pressure">0 mb</span>

</li>

<li class="info wind">

<h2 class="title">RAIN</h2>

<span class='update' id="rain">0%</span>

</li>

<li class="info humidity">

<h2 class="title">HUMIDITY</h2>

<span class='update' id="humidity">23%</span>

</li>

</ul>

</div>

</div>

<script>

setInterval(drawClock, 2000);

function drawClock(){

var now = new Date();

var hour = now.getHours();

var minute = now.getMinutes();

var second = now.getSeconds();

//Date

var options = {year: 'numeric', month: 'long', day: 'numeric' };

var today  = new Date();

document.getElementById("date").innerHTML = today.toLocaleDateString("en-US", options);

//hour

var hourAngle = (360*(hour/12))+((360/12)*(minute/60));

var minAngle = 360*(minute/60);

document.getElementById("hour").style.transform = "rotate("+(hourAngle)+"deg)";

//minute

document.getElementById("min").style.transform = "rotate("+(minAngle)+"deg)";

//Get Humidity Temperature and Rain Data

var xhttp = new XMLHttpRequest();

xhttp.onreadystatechange = function() {

if (this.readyState == 4 && this.status == 200) {

var txt = this.responseText;

var obj = JSON.parse(txt); //Ref: https://www.w3schools.com/js/js_json_parse.asp

document.getElementById("rain").innerHTML = obj.Rain + "%";

document.getElementById("temperature").innerHTML = Math.round(obj.Temperature) + "&deg;C";

document.getElementById("temp").innerHTML = Math.round(obj.Temperature) + "&deg;C";

document.getElementById("humidity").innerHTML = Math.round(obj.Humidity) + "%";

document.getElementById("pressure").innerHTML = Math.round(obj.Pressuremb) + " mb";

}

};

xhttp.open("GET", "readADC", true); //Handle readADC server on ESP8266

xhttp.send();

}

</script>

</body>

</html>

)=====";

WEATHER STATION MAIN CODE

#include <ESP8266WiFi.h>

#include <WiFiClient.h>

#include <ESP8266WebServer.h>

#include <SFE_BMP180.h>

#include <Wire.h>

#include "index.h" //Our HTML webpage contents with javascripts

#include "DHTesp.h" //DHT11 Library for ESP

#define LED 2 //On board LED

#define DHTpin 14 //D5 of NodeMCU is GPIO14

SFE_BMP180 pressure;

#define ALTITUDE 1655.0 // Altitude in meters

DHTesp dht;

//SSID and Password of your WiFi router

const char* ssid = "Alexahome";

const char* password = "hngzhowxiantan";

ESP8266WebServer server(80); //Server on port 80

void handleRoot() {

String s = MAIN_page; //Read HTML contents

server.send(200, "text/html", s); //Send web page

}

float humidity, temperature;

void handleADC() {

char status;

double T,P,p0,a;

double Tdeg, Tfar, phg, pmb;

status = pressure.startTemperature();

if (status != 0)

{

// Wait for the measurement to complete:

delay(status);

status = pressure.getTemperature(T);

if (status != 0)

{

// Print out the measurement:

Serial.print("temperature: ");

Serial.print(T,2);

Tdeg = T;

Serial.print(" deg C, ");

Tfar = (9.0/5.0)*T+32.0;

Serial.print((9.0/5.0)*T+32.0,2);

Serial.println(" deg F");

status = pressure.startPressure(3);

if (status != 0)

{

// Wait for the measurement to complete:

delay(status);

status = pressure.getPressure(P,T);

if (status != 0)

{

// Print out the measurement:

Serial.print("absolute pressure: ");

Serial.print(P,2);

pmb = P;

Serial.print(" mb, ");

phg = P*0.0295333727;

Serial.print(P*0.0295333727,2);

Serial.println(" inHg");

p0 = pressure.sealevel(P,ALTITUDE); // we're at 1655 meters (Boulder, CO)

Serial.print("relative (sea-level) pressure: ");

Serial.print(p0,2);

Serial.print(" mb, ");

Serial.print(p0*0.0295333727,2);

Serial.println(" inHg");

a = pressure.altitude(P,p0);

Serial.print("computed altitude: ");

Serial.print(a,0);

Serial.print(" meters, ");

Serial.print(a*3.28084,0);

Serial.println(" feet");

}

else Serial.println("error retrieving pressure measurement\n");

}

else Serial.println("error starting pressure measurement\n");

}

else Serial.println("error retrieving temperature measurement\n");

}

else Serial.println("error starting temperature measurement\n");

int rain = analogRead(A0);

//Create JSON data

String data = "{\"Rain\":\""+String(rain)+"\",\"Pressuremb\":\""+String(pmb)+"\",\"Pressurehg\":\""+String(phg)+"\", \"Temperature\":\""+ String(temperature) +"\", \"Humidity\":\""+ String(humidity) +"\"}";

digitalWrite(LED,!digitalRead(LED)); //Toggle LED on data request ajax

server.send(200, "text/plane", data); //Send ADC value, temperature and humidity JSON to client ajax request

delay(dht.getMinimumSamplingPeriod());

humidity = dht.getHumidity();

temperature = dht.getTemperature();

Serial.print("H:");

Serial.println(humidity);

Serial.print("T:");

Serial.println(temperature); //dht.toFahrenheit(temperature));

Serial.print("R:");

Serial.println(rain);

}

void setup()

{

Serial.begin(115200);

Serial.println();

// dht11 Sensor

dht.setup(DHTpin, DHTesp::DHT11); //for DHT11 Connect DHT sensor to GPIO 17

pinMode(LED,OUTPUT);

//BMP180 Sensor

if (pressure.begin())

Serial.println("BMP180 init success");

else

{

Serial.println("BMP180 init fail\n\n");

while(1); // Pause forever.

}

WiFi.begin(ssid, password); //Connect to your WiFi router

Serial.println("");

// Wait for connection

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

//If connection successful show IP address in serial monitor

Serial.println("");

Serial.print("Connected to ");

Serial.println(ssid);

Serial.print("IP address: ");

Serial.println(WiFi.localIP()); //IP address assigned to your ESP

server.on("/", handleRoot); //Which routine to handle at root location. This is display page

server.on("/readADC", handleADC); //This page is called by java Script AJAX

server.begin(); //Start server

Serial.println("HTTP server started");

}

void loop()

{

server.handleClient(); //Handle client requests

}

REAL TIME WORKING:

RESULT: